# Swarm

## Getting IP addresses and connecting

To get started, turn on your Tello Talent. If you can't find its Wi-Fi, you'll need to reset it. To do this, turn it off by pressing the power button, then press and hold the button for 5 seconds.

### Connecting to Your Router

- Go to telloconnect.py and update the SSID and password to your router's.

- Run telloconnect.py.

The Tello Talent will then reboot and connect to your router. The Tello's own Wi-Fi will be gone now.

### Finding the Tello's IP Address

- Connect your computer to the same router as the Tello.

- Go to your computer's Wi-Fi settings to find the IP address of both the router and your computer. Write them down.

- Go to networkScan.py, and replace '192.168.0.1' on line 12 with your router's IP.

- Run scan_network() in a Python file.

The first IP address will be the router itself, one will be your computer, and the rest will be the Tellos.

### Updating the Tello Script

- Go to whichever file you want to run, eg facetracking.py or 3_drones_orbit.py.

- Paste the Tello IP addresses into tello_address or tello_addresses_og.

## Programming Tello Swarm

### Installation and Setup

- Install the DJITelloPy library. The documentation is available online.

- Import the TelloSwarm module from djitellopy.

- Create a swarm object by passing the IP addresses of the drones as parameters, like this: swarm = TelloSwarm.fromIps("foo", "bar").

- The swarm object is responsible for controlling all the drones.

### Controlling the Swarm

- To make all the drones do the same thing, write swarm.tellopy_command_here().

- To run commands one at a time, use swarm.sequential(lambda i, tello: tello.command_here(i...)).

- To do a unique action for each drone, use swarm.parallel(lambda i, tello: tello.command_here(i...)).

### Final Steps

- Remember to call swarm.land() to land the drones after executing commands.

- Call swarm.end() to clean up after the program is finished.

## File descriptions

### telloconnect.py

- Run this to connect a tello to your router, rather than having it create it’s own.

- First turn on the tello drone and connect your computer to the tello’s wifi.

- Then paste the SSID and password of your router in telloconnect.py.

- Run telloconnect.py on your computer. The tello should reboot in 3 seconds and turn off it’s own wifi. Then it should connect to your router. Use networkScan.py to make sure it worked.

### networkScan.py

- A function to return all drones connected to your router, given your router’s ip address. It can take several minutes to complete.

- Usage example:

from networkscan import scan_network

drones = scan_network()

### cooldown.py

- Run to cool down an overheating drone. Change the host parameter to the drone’s ip first.

### pid.py

- A class for creating PID controller objects, used in face tracking.

### facetracking.py

- Run this to track your face with a single drone.

- First, connect your drone and computer to the same wifi network.

- Then change tello_address to the drone’s ip address and local_address to yours.

### networkScan.py

- A function to return all drones connected to your router, given your router’s ip address.

- Usage example:

### 3_drones_individual.py

- Run to make all your tello drones do a flip.

- First, make sure your tellos and your computer are connected to the same wifi network. Then add your tello ips into tello_addresses_og and your own ip into local_address. You can have any number of drones, the file name is misleading.

### 3_drones_orbit.py

- Run to make all your tello drones orbit a center.

- First, make sure your tellos and your computer are connected to the same wifi network. Then add your tello ips into tello_addresses_og and your own ip into local_address. You can have any number of drones, the file name is misleading.

- Then change the radius variable to set the radius of the circle the drones are orbiting in. (in cm)

- Place your drones in a circle with their camera facing in towards the center.

- You can also change degrees to be negative to make them orbit clockwise, or increase/decrease degrees to make them orbit more of the circle at a time. Speed can be used to change the speed of the orbit.

  - *Changing degrees of speed changes the time taken for each curve instruction, so you might have to change the delay parameter in the send(instruction, delay) functions as well.

- Run the file to make them orbit.